KGS-GSM-380-2 Remote Control Switch Manual

GSM remote control switch is a very widely used wireless control device.This product can realize mobile phone remote control (Built-in an Internet of Things flow card),switches for various electrical appliances such as water pumps,lamps,motors, etc..315MhzRF remote control,and mobile phone can view the status of the device in real time,has the advantages of easy installation,simple use,safe and reliable.

1. Technical index

1. Rated voltage: 185V-450VAC.

2. Output mode: Control contactor coil

3.RF frequency: 315MHz,special frequency can be customized.

4.Control mode: Mobilephone APP/RF remote control/manual.

5. Working mode: self-locking/interlocking (Mobilephone APP can only be controlled by self-locking,the remote control can choose self-locking or interlocking)

6. Remote control learning method: Long press the OFF button for more than 5 seconds,release,the code indicator light flashes,press the remote control button,the code indicator light goes out,learning is successful. (Note: If the optional remote control is selected,it is shipped from factory,the remote control and receiver have been paired,users do not need to operate,you can use it after connecting the cable!).

7. Remote code clearing method: Long press the ON button,and then the receiver is powered on,more than 5 seconds,the indicator flashes and goes out,the code is successfully cleared.

8. Control distance: It varies according to the matching remote controller.It can be controlled where there is a mobile signal when the mobilephone APP is controlled.

9. Unbind the APP: Long press "My Smart Switch" for 2 seconds to confirm unbind the device.

2. Steps for usage

1. Connect the wires strictly according to the wiring diagram and install the antenna!

2. Open WeChat-scan the QR code of Kaige Internet of Things-authorize login-scan to add-scan the QR code on the product or add it manually Enter the 15-digit IEMI code below the QR code.

3. After completing the addition,wait for 1 minute and the green indicator flashes.

3. Pin and connection

Warning: This product is a strong power product,please do not operate it by non-professionals,otherwise you will be at your own risk!

380VAC input

Fire Wire input

Power Indicator

Code indicator

Status Indicator

Interlock

Self-locking

Signal indicator

Manually open

Manually off

Working mode conversion

Fire Wire output

380VAC output

- Precautions for use

- Please read the instructions carefully before installation,please strictly follow the wiring diagram wiring,do not operate live wiring,otherwise you will be at your own risk.

- Working mode: Self-locking: control with 1 button,press the remote control button once the receiver turns on,and press the remote control button again to disconnect the receiver.

- Interlock: Control with 2 buttons,press the first button to open,press the other button to close.